Fall 2020

ECET 209

Assessment #R

Digital Inputs and Outputs

| Student Name | Student ID |
| --- | --- |

| S | Topic | Maximum Points | Earned Points | Feedback Comment(s) |
| --- | --- | --- | --- | --- |
| 1 | C code and simulation for Digital Output | 25 |  |  |
| 2 | C code and simulation for Digital Output | 25 |  |  |
| 3 | C Code: Digital Input and Output | 25 |  |  |
| 4 | C Code: Digital Input and Output | 25 |  |  |
| 5 | Bonus | 5 |  |  |
|  | Total | 100 |  |  |

NOTES:

•	Hand written reports will not be accepted (except if requested by Instructor)

•	The assessment is out of 100 marks

•	Up to 20 marks off for untidy / incompliant reports

- C code and simulation for Digital Output                                        (25 Points)

- Analyze the following C Language program

Explain in detail each code line using comments                                      (15 p)

void main() { /** TRISB is used to set the port direction PORTB is used to read/write the port pin values **/ TRISB=0x00; // Sets PORTB as the output PORTB=0x00; // Toggle PORTB off TRISD=0x00; // Sets PORTB as the output PORTD=0x00; // Toggle PORTD off while(1) { PORTB=0xF0; // State 1 – lower 4 bits in PORTB for output PORTD=0X0F; // State 1 - – lower 4 bits in PORTD for output Delay_ms(2000);// Delay for 2000 milliseconds PORTB=0x03; // State 2 – call write function for output PORTD=0XC0; // State 2 Delay_ms(2000); // Delay for 2000 milliseconds } }

- Simulate the code using PICSimLab and paste 2 screenshots showing states

(5p each)

First Screenshot

Second Screenshot

- C code and simulation for Digital Output                                        (25 Points)

- Analyze the following C Language program

Explain each code line using comments                                                                                     (17 p)

int mask(int character) { switch (character) // decide upon the following condition based on the character { case 0 : return 0x01; // return hex value of 1 case 1 : return 0x08; // return hex value of 8 case 2 : return 0x06; // return the hex value of 6 case 3 : return 0x30; // return the hex value of 3 } } void main() { int a, x; // declare two variables ‘a’ and ‘x’ // set port A as the output and toggle it to ON TRISA = 0x00; PORTA=0x00; // set port D as the output and toggle it to OFF TRISD = 0x00; PORTD=0x00; while(1) { PORTA=0x20;// set port A to 32 bits for(x=0;x<=3;x++) { PORTD=mask(x); // set port D to a masked value of the loop counter x Delay_ms(2000); // delay for 2000 milliseconds } PORTA=0x10;// set port A to 16 bits for(x=0;x<=3;x++) { PORTD=mask(x); // set port D to a masked value of the loop counter x Delay_ms(2000);// delay 2000 milliseconds } } }

- Simulate the code using PICSimLab and paste 8 screenshots (Also draw accepted)                                                                                    (8p/Each 1)

First part Second part

- C code and simulation for Digital Input                                         (25 Points)

Write code for PIC 16F877A using C language to light PORTD bits D2, D3 and D4 if button PORTB bit4 is pressed.

When button released, all LEDs should turn off                 (15 p)

Each code line should be explained using comments     (10 p)

void main(){ TRISB.F3 = 1; // set 4th bit of PORTB as input TRISD.F1 = 0; // set 2nd bit of PORTD as output TRISD.F2 = 0; // set 3rd bit of PORTD as output TRISD.F3 = 0; // set 4th bit of PORTD as output // turn the LEDs OFF PORTD.F1 = 0; PORTD.F2 = 0; PORTD.F3 = 0; // in a while loop light all the LEDs while(1) // endless while loop { if(PORTB.F3 == 0) // if the switch is pressed { Delay_ms(100);// delay for 100 milliseconds – switch debouncing if(PORTB.F3 == 0) // if the switch is still pressed { // turn the LEDS ON PORTD.F1 = 1; PORTD.F2 = 1; PORTD.F3 = 1; Delay_ms(2000);// delay for 2 seconds // turn the LEDs OFF PORTD.F1 = 0; PORTD.F2 = 0; PORTD.F3 = 0; } } } }

- C Code: Digital Input and Output                                                    (25 Points)

Write code for PIC 16F877A using C language to light figure    on all 7 segments if button PORTB bit3 is pressed.

When button released, all 7 segments should turn off                 (15 p)

Each code line should be explained using comments     (10 p)

void main(){ TRISB.F2 = 1; // set 4th bit of PORTB as input TRISD = 0x00;// set PORTD as the output // turn the whole segment OFF PORTD = 0x00; // in a while loop light all the LEDs while(1) // endless while loop { if(PORTB.F2 == 0) // if the switch is pressed { Delay_ms(100);// delay for 100 milliseconds – switch debouncing if(PORTB.F2 == 0) // if the switch is still pressed { // turn the segment ON PORTD = 0X7F;// 0x7F is the hex value of decimal 8 Delay_ms(1000);// delay for 2 seconds PORTD = 0x00; // turn the segment off } } } }

Bonus Question:                                                                                 (5 points)

Analyze the following C Programming Language instructions and answer the questions:

void main() { TRISB = 0x00; PORTB=0x00; while(1){ PORTB=0x03;//state 1 Delay_ms(1500); PORTB=0x05;//state 2 Delay_ms(500); } }

- What is the duration of state 1 in seconds?  1.5 seconds

- What is the duration of state 2 in seconds?  0.5  seconds